By Joseph Padfield

November 1999

Hamilton Kerr Institute

The Procedure for Mosaicing IR images, using the Torch Computer.

Table of contents:

1.	Introduction:	3

2.	Turning on the Torch Computer:	3

2.1.	Start up	3

3.	Infra-red imaging software:	3

4.	Preparing to Mosaicing IR images:	5

Removing Entries from Columns:	5

4.1.	Loading images:	5

4.2.	Looking it at images:	6

5.	Mosaicing:	6

Selecting and removing mosaicing points:	6

Mosaicing:	7

Common, Simple Mosaicing problems:	7

5.1.	Workspaces and Their Structure:	8

Structuring Your Workspace:	9

5.2.	Balancing the contrast of the image:	11

5.3.	Converting your finished image:	11

5.4.	Transferring your image and printing:	12

6.	Problems with Mosaics:	12

7.	Continuing Previous Work:	13

8.	Process Summary:	14

Selecting and removing points for mosaicing:	14

Mosaicing:	14

Transferring files from Linux to windows:	15

9.	Shutting Down The Computer Or Returning to Windows 98:	16

## Introduction:

This set of instructions is intended to explain the basics instructions required to, turn on and use the Torch computer and then to use it to mosaic images which have been captured on the Sun compute.  The instructions manuals supplied with the IR software go into much greater detail and even explain how the program actually works, so if you require more information than is supplied in this report please consult the software instructions manuals.

Information for Capturing IR images using the Hamamatsu camera and the SUN computer are discussed in another document.

## Turning on the Torch Computer:

### Start up

The Torch computer is set up to allow you to use two different operating systems, Windows 98 or Linux.  When the computer is turned on it will stop its booting up sequence to allow you to tell it whether to load Windows 98 or Linux.  If you just leave the computer, or press return it will load Windows 98.  To load Linux, you will have to type Linux and press return.

- Plug computer in and turn on.

- To use the IR software, load Linux by typing Linux and pressing return, when the booting sequence pauses.

- A login window will appear, then enter user name ‘general’ and the password ‘fun@linux’, when asked to.

- A windows style screen will appear with several windows open, the exact number will depend on what was open when the computer was previously shut down.

- It will always start with the IR software window and a file manager type window open (though the files manager will probably appear under the IR software window).

- In addition there may be a ‘Bash’ window, for entering text commands and another file manger type window.

## Infra-red imaging software:

Begin by maximising the window, this can be done by CLMB on the small square in the top right corner of the ip window.

On the ruler at the top of the ip window, HRMB on an area free from words and then move the arrow down to ‘infrared’ on the menu which will appear then let go of the mouse button.

THIS WILL SIMPLIFY THE WINDOW. DON’T WORRY, THE WORDS ARE MEANT TO DISAPPEAR.

The ip window with the infrared menu list, will have the following headings: [File] [Edit] [Balance] [Dialogs] [Grab] [Mosaic] and [Shrink].  By CLMB once over these headings menus will be produced to perform commands or move through to further sub menus, which can again be accessed by simply CLMB over the entries in the new menus.  To simplify these instructions the commands performed within the IR software will be indicated as follows:

[Window] [ruler entry: menu entry: etc.]

For example: [ip Window] [File: New: Column]

This will indicate that, from the ip Window ruler bar you should CLMB once on the ‘File’ heading and then CLMB once on the ‘new’ entry in the drop down menu and then finally CLMB once on the ‘column’ entry in the sub menu.

It should be noted at this point that this piece of software is quiet complex and is capable of doing many things.  The following instructions only cover the basics of capturing and mosaicing images, for further information please consult the software manuals.

NB: In order to organise your work, while using the IR software, the programmer has designed a column system.  All information, which the program is considering, is listed in smaller special windows within the ip window called columns. The columns produced by this program are each given a letter, the first column being ‘A’, the next ‘B’ and so on.  Entries within these columns, such as video windows, images mosaicing instructions etc. included an accessing button, the name and a description of the entry. In order to access any entries in a column simply DLMB over the button on the left of the entry.  Large number of columns can be opened, using the command [ip Window] [File: New: Column], so that you can organise your images and calculations. Further information regarding columns will be discussed during the mosaicing instructions, see section 4.

## Preparing to Mosaicing IR images:

This section is intended to explain the basic process through which a selection of images can be made into a mosaic.  The ip software is capable of doing various things and this document is not intended to explain the full usage of the software.  Feel free to read the provided manuals if you really want to learn more.

To mosaic images you only need the computer on, so you can leave the camera, monitors etc all off.  Please refer to section 2 for an explanation of how to turn on the computer.   The basics for mosaicing are explained first, but it is good to read section 5.1 before you actually start loading images and sticking them together.

Also you do not need to mosaic all of your images together at one time.

As long as you plan your work well you can carry out the mosaicing procedure whenever you have spare time.  However it can be easier to follow your work through if you can do it in one go.

- Turn on the computer as described in section 2.

When you start to mosaic a set of images you are going to need to load them into the software and then begin to instruct the computer as to which pictures need to be stuck to which.  This is all done via the column windows, which were first introduced in section 3, but are now going to be discussed again.

#### Removing Entries from Columns:

At any time during your work you may want remove entries from your columns, this can be achieved as follows.

- First select the entry button or buttons that you want to delete.

Selecting and de-selecting entries in your columns can be done simply by CLMB over the button associated with the particular entry.  In order to select more than one entry at the same time you will need to hold down the ‘shift’ key while you CLMB on the entry buttons.

- To remove the selected entries, hold down the ‘control’ key and press ‘r’, i.e. press ‘Ctrl-R’.

### Loading images:

Images can be loaded into the software and added to a column using the following commands:

- [ip Window] [File: Load: Image]

This will open an image-loading window, the load window contains information relating to which directory you are looking into, which files are present and the type of file the program is currently trying to save.  Once you have made sure that you are in the right directory your images can be loaded into your column, simply by DLMB on the name of the image in the file list.  NB: You may want to move the load window so that you can see the column into which you are loading the images, this way you can see when they appear.  You can often DLMB the image name correctly but the image will not be loaded into the column, this is often to do with the speed at which you click the mouse buttons.  Do not worry about this; just make sure each of the images you want to load appears in the column as you DLMB on the image names in the loading window.

NB: If you are loading a selection of files ensure that the ‘pin me up’ box has been ticked.  This simply stops the load window from closing each time you load an image.

Once you have finished loading the images you require at that point, CLMB on ‘cancel’ in the load window to close it.

### Looking it at images:

Once you have loaded images into your column you can view them in an image window simply by DLMB over the button to the left of the specific image entry in the column.  To make the mosaicing process more straight forward you may want to play with the magnifications of the images, ( [image window][View:Magnification:*] )

## Mosaicing:

This actual process of mosaicing is quite straightforward; images are stuck together in pairs to produce larger images which are then stuck together in pairs and so on.

Too mosaic two images together open viewing windows for both of the images and move them so that you can see both images at the same time.  NB: it is often easiest if you orientate the images so that the edges you wish to overlap are next to each other.  Next you need to help the computer by giving it a place to start when it tries to stick to two images together.

#### Selecting and removing mosaicing points:

Look at the two images and find a point, which is present on both; this can be a dot, the end of line, a patch of white, etc. it does not matter as long as it is present on both images.  Once you have found your point move the mouse arrow to your chosen point in the first image and CRMB, then do the same for the second image.  (NB: the order in which the points are chosen does not matter as the computer can work out which is to go were).  When you CRMB over your points a small cross will appear with a nametag.  These selected points are called regions and they are entered into you current column as with the images, with a button, description, etc.

If you find you have placed the points in the wrong place they can be moved by HLMB over the nametag and just moving it to the position you require.

Selected points/regions can be deleted either by removing the entry from the column or by DLMB over the actual nametag, this produces a small window with information relating to the selected point.  One of the options in this window allows you remove the point.

#### Mosaicing:

Once you have selected your overlap points the two images can be mosaiced using one of the following two commands:

- [ip window][Mosaic: Left_right_mosaic]

- [ip window][Mosaic: Top_bottom_mosaic]

Your choice between these two commands is obviously dependent on whether the images are to go together top and bottom or left and right.

Either of these commands will cause the computer to stick the two images together and enter the resultant image as the next entry in your column.

Due to the mathematics used by the IR software, each join will on average cause a positioning error of about 0.5 pixels, even if your input images contain no geometric distortion. If there are distortions in your input images (there usually are distortions in infrared images), then errors can be as much as 1{2 pixels per join. These errors accumulate, as the number of images you join becomes larger.

If you join a strip of 10 images together with Mosaic left right, on average you can expect a total error of about 5 pixels. If you join two strips like this together top-bottom, you can therefore expect a mismatch of about 2.5 pixels at each end of the join.

You can minimise the effect of these errors if you assemble your images differently. Suppose you have a 10 by 10 mosaic to build. Instead of making and joining 10 strips of 10 images each, make four 5 by 5 sub-mosaics (one for each quadrant) and then join these four quadrants together. The errors will now be more evenly spread over the image and therefore will be less visible.

#### Common, Simple Mosaicing problems:

- The computer will sometimes complain that there is insufficient overlap to mosaic your two images.

This can often be sorted by moving your points to a different overlap position.  It is good to have the points roughly the same distance from the edge on both images.  Also if you are trying to mosaic bigger images, which are themselves composed of many smaller images, the edges of the images may not be straight so you need to ensure that you have sufficient overlap right along the edge of the image.

This error message has also been produced in another situation.  If you have tried the above solution without success check the co-ordinates of the region.  If the numbers are too high and the computer does not register the co-ordinates when you move the mouse arrow across the image you will need to clone the image.  This can be done in any of your columns but it might be good to open a new column just to fix this problem.  Select the column into which you would like the new version of the image to appear.  HRMB over the button of the specific image and then choose ‘clone item’ from the menu which appears.  You should then be able to carry on mosaicing your images using the cloned image rather than the original.

- The computer may inform you that it needs 2 arguments rather than one.

This just means that you have accidentally selected one of the entries in your column and you need to de-select it or select the other region if you had deliberately tried to select your regions.  Selecting and de-selecting entries in your columns can be done simply by CLMB over the button associated with the particular entry.  In order to select more than one entry at the same time you will need to hold down the ‘shift’ key while you CLMB on the entry buttons.

- Strange images or images with lines or marks can be produced if your selected points do not match correctly, or if you choose the wrong command, either left_right or top_bottom mosaic.

If your points do not match up properly you can just move them as described before and the computer will automatically correct the mosaiced image.  If you have already closed the images so that you can no longer see the positions of your points you can just re-open them by DLMB over the button next to the entries for the two regions in question.

If you have used the wrong command you will have to remove the incorrect image and them use the correct command.  In this situation the computer will them just mosaic the last two regions in your selected column, thus producing the correct image.

### Workspaces and Their Structure:

All of your columns, with the commands and images are known as a ‘workspace’, like any other working area unless you keep it ordered you would soon loose track of what you are doing.  Keep track of your work, in order to do this it is good to name each of your columns clearly so that you will remember what they contain.  The structure for planning your workspace, which is going to be described below is not the only way of structuring your workspace, but it, seems to work.  Once you have the hang of the software you can alter the structure to suit your own needs and requirements.

One of the first things to remember is that computers are temperamental, particularly, so save your workspace regularly.  This is accomplished by using, [ip Window] [File: Save: Workspace].   This will open a save window similar to the one encountered during the image saving process.  Again simply give the workspace a name, followed by the suffix ‘.ws’, ensure you are saving in the right directory and then just save the workspace.

Within the save window there are several entries and these have been explained below.

- The ‘PATH’ denotes were on the computer the images will be saved.

- The ‘FILTER’ denotes the type of file that the computer will save the image as.

- ‘DIRECTORIES’: list the subdirectories present within your current folder.

- ‘FILES’: list the files present within your current folder.

DLMB on the name of your directory, the ‘PATH’ should then read /home/general/’X’ where X is your directory.   If you have created another subdirectory within this directory you will now have to DLMB on it, to change the ‘PATH’ to include it.

In the bar, entitled ‘selection’ at the bottom of the window  ‘untitled.ws’ should be written.  The text in this box shows that name the computer will give to an image when you save it.  By CLMB over the bar a flashing cursor will appear next to the text and you can alter the name.   NB: the name you designate for your image must end in ‘.ws’

NB: when you try to re-save a workspace with the same name as before the program will ask you if you want to overwrite the previous version.  In this situation you can normally just click ok.  If you have made major changes, which you are not sure of and do not want to delete the last workspace, you saved simply give the second version a different name.

#### Structuring Your Workspace:

Obviously the structure of your workspace will alter depending on the number of images you have to mosaic, but here the structure given will be for a mosaic comprised of 4 rows and 6 columns, a total of 24 images, see figure 2.

When the program starts up you will begin with one empty column.  If you or someone else, have already been working with the software it is good to save your previous work and then delete all of the existing columns and then open a fresh empty one to begin your mosaicing.

1          2          3          4          5          6

R1

R2

R3

R4

gy

Figure 1: Diagram showing the numbering and positioning of the images in a 4 x 6 mosaic.  The size of one whole image, image R1.1, has been marked with diagonal lines.

You should begin by loading images to you first column, but not all of them, in this case just the six images in the first row.  You can then re-name your column to be ‘R1-images’.  You then want to open a new column, before the column appears you will be presented with a small window in which you can click on the ‘comment’ line and give a name to the column.  Name this second column, ‘R1:1-3&4-6’ i.e. in this column you will be mosaicing two sets of three images, images 1-3 and then images 4-6.

Now with column ‘R1:1-3&4-6’  selected you then want to open up images 2 & 3 from R1 and assign your regions, then mosaic them together.  This will produce a total of three entries in your second column.  Once this has been done you want to open up the image produced from joining 2&3 and then mosaic it to image 1.  You will now have 6 entries in you column.  You can then repeat the process for images 4-6, i.e. mosaic 5 to 6 and then mosaic the resultant image to image 4.

When you are not using columns it is good to minimise then by CLMB on the black triangle in the top right corner of the column.  You can even move the minimised columns up into nice rows down the left side of your ‘ip window’, though this is up to you.

You can then repeat this process for each of the three other rows, producing columns named; ‘R2-images, R2:1-3&4-6, etc.

You know want to open another new column, call this one, ‘1-3:R1&R2’.  Now with this new column selected open up the mosaic of images 1-3 from R1 and images 1-3 from R2.  These two images should, ideally be the ninth entries in the rows named, ‘R1:1-3&4-6’ and ‘R2:1-3&4-6’, though you may have to check this.  The two images can then be mosaiced together, this will actually complete the upper left corner of the complete image.  Once this is done open up new columns and complete the other three corners of the painting, naming these columns, ‘4-6:R1&R2’, ‘1-3:R3&R4’, etc.

Once you have completed all of the four corners you can then open a final column and call it ‘Mosaic’.  With this column selected you then want mosaic the final image in ‘1-3:R1&R2’ with the final image in ‘1-3:R3&R4’, to produce the left half, and then similarly produce the right half.

The complete mosaic can then be produced by mosaicing the two new images you have produced in your ‘Mosaic’ column together.  This should produce your completed IR mosaic.  This column naming system will obviously need to be extended or altered depending on the number of image, but keep it clear and concise so that you can follow your own work and be able to make corrections were required.

### Balancing the contrast of the image:

Once you have a completed mosaic the last thing you have to do with IR software is to balance out all of the contrast differences that you may get from image to image.

- [ip window][Balance:Global_mosaic_balanch_search]

Again for large mosaics this will take a long time, as the computer is going back to the original images and then comparing them as they are put together and working out the overall contrast.

If you are still alive and have produced a complete mosaic without killing yourself, a colleague or more likely the computer, WELL DONE.

### Converting your finished image:

Once you have your completed and saved image it will be in a vips format, i.e. *.v, in order for it to be printed out it needs to be converted to a ‘tif’ format.

Minimise or close your ‘ip window’ and then move the mouse pointer over to the ‘cmdtool’ window:

- Type pwd and press return this command will check which directory you are in, you can then use the ls command to check that your image was saved in this directory.

- If you need to change directory, type in cd /home/general/**1/**2, (where **1 is the name of your own directory and **2 is the name of the subdirectory in which you have saved your images), and press return.

- You can again check with ls to ensure your file is present.

When you are sure you are in the right directory you can continue as follows:

- Type in: im_vips2tiff *.v **.tif none and then press return.  (where * is the name of your image and ** is the name you want the new version of the image to be.  NB: these names can be the same if you want.

The computer will now have produced your tif image.

### Transferring your image and printing:

Your tif file now needs to be manipulated and printed.

Type cp command in the bash window, to move file to /mnt/d which will then be accessible as drive d in windows98.

## Problems with Mosaics:

- You have produced you mosaic but it looks all patchy, with squares of light and dark.

This should not be a problem, as it will be normally be solved using the balancing commands.

- There seems to be an error or errors in the image, normally seen down the edge of a painting but possibly in the centre.  This can be the result of two separate problems.

- Mosaicing problems: One or more of your joins, though they appeared ok before, are not quite correct.  This will probably be the case if you can see something similar to examples given in figure .

Figure 1: Schematic of some of the possible mosaicing problems, obviously the real problems may be more subtle that this.

To solve this type of problem you need to go back to your naming grid pattern, figure 2, and work out which images or group of images are wrong.  Once you have found this you need to go back to your workspace, find the correct column, and then the two regions you picked which resulted in the problem. DLMB on the buttons associated with these two regions, in order to open up the images.  You can then simply move the regions, by HLMB and dragging the region a new location.  NB: Each time you move a region the computer will recalculate all of the mosaics that it produced after that one, this may take some time, you will need to be patient.  You will probably have to move two regions, to correct each problem, though you may get away with just moving one of them.

- Mathematical Problems: This one is slightly harder to fix. The curvature of the IR tube, the angle of the painting on the easel or even the positioning of the camera stand, can lead to dimensional irregularities in the captured images.  When the computer joins images it does various mathematical things to the edges to achieve the join and this can magnify or build up some of the dimensional irregularities, as the images are mosaiced.  With this type of problem you can end up with slightly different problems, see figure 4.

Figure 1: Schematic of some of the type of problems caused by mathematical/dimensional errors, obviously the real problems may be more subtle that this.

A

B

This type of problem has been solved in previous cases by increasing the overlap, when mosaicing the larger section.  For example, if you were mosaicing 24 images, as shown in figure 2, using the workspace structuring suggested, problems A and B can be solved as follows.

Once you have produced all four, corner segments of the mosaic you will need to produce additional segments to overlap the problem join.  With ‘A’, in figure 4, you would also produce mosaics of  ‘1-3:R2&3’ and ‘4-6:R2&3’ and then join the respective corner segments on to the tops and bottom, to produce the two completed halves before you can join them to produce the completed mosaic.  Obviously with situation ‘B’ you would need to produce ‘3-4:R1&R2’ and ‘3-4:R3-R4’ instead.

NB: Remember, that if you did not capture the images yourself and you are not sure that the painting was completely flat during the capturing procedure it may not be possible to produce a perfect mosaic.  So don’t spends hours trying to solve a problem which you might not be able to fix.

## Continuing Previous Work:

In order to continue to mosaic a set of images, that you have already begun but not finished, you will need to reload in your workspace.  Turn on the computer and login as described before, open up or select the vips software widow and reload your previously saved workspace as follows:

- [ip Window] [File: Load: Workspace]

You can now continue on from were you left off mosaicing the rest of the image, as described in Chapter 4.

## Process Summary:

##### Turning on the Computer:

- Plug the computer in and switch on.

- Type in Linux at the ‘LILO Boot:’ prompt.

- Enter user name ‘general’ and the password ‘fun@linux’, when asked to.

- This will automatically open the mosaicing software.

##### Creating your own folders:

- Produce your own folder in General and then sub-directories or folders for each painting or project you examine.

- [file manager type window][CRMB, File:New:Folder]

Name your folder.

##### Procedure for Mosaicing IR images:

- Turning on the computer, as described above, (and in section 2) will open the mosaicing software.

##### Loading images:

- Images can be loaded into the software and added to a column using the following command:

- [ip Window] [File: Load: Image]

- The images can then be viewed by DLMB on the associated button.

- Entries I columns can be removed by selecting them and then pressing Ctrl-R.

#### Selecting and removing points for mosaicing:

- Open up the two images to be joined, examine them and then select the overlap region by CRMB.

- These selected points can be moved by HLMB over the point and dragging it to the new position.

- Regions can be removed by DLMB one the region and them choosing the remove option from the pop up menu.

#### Mosaicing:

- When you try to mosaic images the computer will always try to combine the last two entries in the selected column, unless you manual indicate which entries you want it to join together.

- [ip window][Mosaic: Left_right_mosaic]

- [ip window][Mosaic: Top_bottom_mosaic]

##### Workspaces and Their Structure:

- Take care with your workspace and save it often.

- [ip Window] [File: Save: Workspace].

##### Loading Workspace:

- [ip Window] [File: Load: Image]

##### Balancing the contrast of the image:

- Once you have a completed mosaic the last thing you have to do with IR software is to balance out all of the contrast differences that you may get from image to image.

- [ip window][Balance:Global_mosaic_balanch_search]

##### Saving your final image:

- Once you have opened up your completed image use the following command to save it.

- [image window][File:save:image]

##### Converting your File:

- Move the mouse pointer over to the ‘cmdtool’ window:

- Type in cd /home/general/**1/**2, (where **1 is the name of your own directory and **2 is the name of the subdirectory in which you have saved your images), and press return.

- Use the ls command to check that your images are in this directory.

- Type in: im_vips2tiff *.v **.tif none and then press return.  (where * is the name of your image and ** is the name you want the new version of the image to be.

#### Transferring files from Linux to windows:

At this time this process can only be done with a root password, therefore go and ask Joe.

## Shutting Down The Computer Or Returning to Windows 98:

- CLMB on the ‘K’ button in the BLC of the screen and choose logout from the menu.

- Click ‘Logout’ on the window that appears.

- This will then take you back to the logout screen.

- Click the ‘Shutdown’ button.

- Select the ‘Shutdown’ option and click Ok.

- You then wait for the computer to shutdown, when it has finished you will see “reached” in white text in the bottom right hand corner of the screen.

- You can then turn the computer off, or press the restart button to start the computer again.

An operating system is the software which runs the actual computer.

When the program first opens there will be no entries within the column so do not get upset if you can not see any.

These buttons appear as slightly raised rectangles down the left of the column and each is given a letter and a number.  The letter is specific to the column and the number just indicates the order in which the entries have been opened, i.e. the first entry in column A will be numbered ‘A1’, etc.

Columns can be renamed at any time simply by DLMB on the top section of the column, typing in  the new name and then pressing return.

Each of the images, you will be examining, are composed of little dots.  The computer counts the little dots and uses them as a way of navigating around the images, i.e. a certain point may be 235 points up and 1024 points in from the edge.  You can seen the co-ordinate numbers being displayed by the computer, as you move the mouse cursor over an image, at the top left of the image viewing window, in the format (123 x 34).  The third number is related to how light or dark each particular area is.

This will indicate that, from the ip Window ruler bar you should CLMB once on the ‘File’ heading and then CLMB once on the ‘save’ entry in the drop down menu and then finally CLMB once on the ‘workspace’ entry in the sub menu.

It is very good practice to delete/remove any extra regions/images, which may be produced by accident or through experimentation, in order to keep your columns as straight forward as possible.  NB: If you remove entries from columns which are involved with the mosaicing of other entries, either in the same column or in different columns you will start to produce error messages and things will get confusing, so be careful.

Format here just denotes the type of file, images can be stored on computer in various ways the most common being, JPEG, tif, and bitmap.  You do not really have to worry about this though.