Rage 128 Programmer’s Guide Supplement

Video Decoding with the Rage Theatre

# INTRODUCTION

The Rage Theater’s Video Decoder is a full featured video decoder for consumer video and multimedia applications. It incorporates all the processing necessary to decode an analog video signal into a digital component stream.  Rage Theatre’s adaptive comb filter and advanced on-chip circuitry ensures that composite and S-Video analog inputs in NTSC, PAL and SECAM format are converted into high quality, low noise digital video for a sharper image.  Features include:

- integrated high-quality horizontal and vertical downscalers

- digital passthrough mode allows full screen TV display of the S-Video or composite input

- converts S-Video or composite inputs to ITU-656 digital video

- supports square  pixel, ITU non-square pixel, and arbitrary scaling

- VBI data capture supports Intercast, Teletext and Closed Captioning decoding.

- dual A/D converters for composite and S-Video support

- advanced adaptive comb filter for better picture quality

- unique noise reduction circuits for clearer video and better compression.

The diagram below illustrates the signal flow through the Video Decoder. The analog front-end stage performs 8-bit A/D conversion, clamping, and automatic gain control (AGC). The comb filter separates the digital composite video into luma and chroma. The Luma Processor removes the sync and setup (only for NTSC) from the combed-luma, and inserts the brightness and contrast levels. The Chroma Processor demodulates the combed-chroma to generate the video components Cr, and Cb. The HSYNC PLL and VSYNC PLL provide line and frame locking to the input video signal.

# 1.0 Initializing the rage theatre for video decoding

To initialize the Rage Theatre for video decoding, there are 5 basic steps that must be performed.  They are:

- Program the video in PLL to the appropriate value (based on the standard), take the PLL out of reset and attach the PLL output to the video in block.

- Program the video decoder registers based on the selected video standard, and selected comb filter type.

- Set the DVS Port to output mode by setting register field DVS_DIRECTION in DVS_PORT_CNTL.

- Set ADC_PDWN in register ADC_CNTL to 0 to power-up the ADC block.

- Take the video decoder, video scaler and DVS port block out of reset by clearing the appropriate bits in register MASTER_CNTL.

## 1.1 Initializing the Video IN PLL

Upon reset, the Rage Theatre is in a primarily dormant state.  Therefore, to use the video decoder, we must first initialize the video in PLL (VIPLL), and then take the video in block out of reset.  To initialize the VIPLL, we must program the VIN_PLL_CNTL register with values to produce a sampling frequency that is required for the given standard.  The equation that determines the sampling frequency generated by VIPLL is:

Sampling Frequency (Fsamp) = Reference Frequency (Fref) * N / (M * P)

Where:

- Fref is the value of  the installed crystal.  This can be determined by looking at the Hardware Configuration Table in the BIOS, specifically byte 7, bits 4:6.

- N (feedback divider) is the VIN_N0 field of VIN_PLL_CNTL, where 1 ≤ N ≤ 2047

- M (reference divider) is the VIN_M0 field of VIN_PLL_CNTL, where 1 ≤ M ≤ 2047

- P (post divider) is the VIN_P field of VIN_PLL_CNTL, where 1 ≤ P ≤ 16 (*note – to set a value of 16, write 0 to this field)

The following table shows the sampling frequencies required for a given standard:

| Video Standard | LinePeriod (S) | Fsamp (ADC sampling frequency) |
| --- | --- | --- |
| NTSC-M, Japan | 63.5555625 | 28.63636MHz |
| NTSC-443 | 63.5555625 | 35.46895MHz |
| PAL-M | 63.49206349 | 28.63689192MHz |
| PAL-N | 64.0 | 28.65645MHz |
| PAL-N combination | 64.0 | 28.65645MHz |
| PAL- BGHIDKK1L | 64.0 | 35.46895MHz |
| PAL-60 | 63.5555625 | 35.46895MHz |
| SECAM-BDGHKL | 64.0 | 35.46895MHz |

Based on a Fref of 29.49 MHz, the following table shows the appropriate values to be used for VIN_PLL_CNTL:

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VIN_M0 | 0x39 | 0x39 | 0x23 | 0x2C | 0x0E | 0x23 | 0x0E | 0x0E | 0x0E |
| VIN_N0 | 0x14C | 0x14C | 0x088 | 0x12B | 0x065 | 0x088 | 0x065 | 0x065 | 0x065 |
| VIN_P | 0x06 | 0x06 | 0x07 | 0x07 | 0x06 | 0x07 | 0x06 | 0x06 | 0x06 |

Once VIN_PLL_CNTL has been programmed with the appropriate values, the PLL must be taken out of reset.  This is done by setting VINRST @ PLL_CNTL1 to 0.  At this point, the video in clock must be switched from the reference clock to the output of VIPLL, since the VIPLL has now been programmed with the proper values.  To do this, set VIN_CLK_SEL @ CLOCK_SEL_CNTL to 1.

## 1.2 PROGRAMMING THE VIDEO STANDARD

Once the VIPLL has been set and enabled, the registers relating to the video standard being used must be set up.  These registers basically define the type of input signal the Rage Theatre will be decoding.  There are eight components that must be set up:

- Standard Select Register and Clamp Control

- Luma Processor

- Chroma Demodulator

- VSYNC PLL

- Clipping Engine

- Comb Filter

- HSYNC PLL

- Timing Generator

Below is an abbreviated version of the RT_SetStandard () function that is included with the sample source code that accompanies this document.  It only covers the NTSC-M standard, where the actual function covers NTSC-M and J, PAL, and SECAM.

void RT_SetStandard (WORD wStandard)

{

double dbFsamp=0, dbLPeriod=0, dbFPeriod=0;

WORD   wFrameTotal = 0;

// Get the constants for the given standard.

GetStandardConstants (&dbLPeriod, &dbFPeriod, &dbFsamp, wStandard);

wFrameTotal = (WORD) (((2.0 * dbFPeriod) * 1000 / dbLPeriod) + 0.5);

// Reset the PLL before setting the standard registers.

WriteRT_fld (fld_VIN_CLK_SEL, RT_REF_CLK);

WriteRT_fld (fld_VINRST, RT_VINRST_RESET);

RT_SetVINClock (wStandard);

WriteRT_fld (fld_VINRST, RT_VINRST_ACTIVE);

WriteRT_fld (fld_VIN_CLK_SEL, RT_PLL_VIN_CLK);

// The VIPLL is now reset, begin programming the registers/fields that

// affect the video standard

// Set the appropriate standard

WriteRT_fld (fld_STANDARD_SEL,     RT_NTSC);

// Set the clamp control

WriteRT_fld (fld_SYNCTIP_REF0,     RT_NTSCM_SYNCTIP_REF0);

WriteRT_fld (fld_SYNCTIP_REF1,     RT_NTSCM_SYNCTIP_REF1);

WriteRT_fld (fld_CLAMP_REF,        RT_NTSCM_CLAMP_REF);

WriteRT_fld (fld_AGC_PEAKWHITE,    RT_NTSCM_PEAKWHITE);

WriteRT_fld (fld_VBI_PEAKWHITE,    RT_NTSCM_VBI_PEAKWHITE);

// Program the Luma processor

WriteRT_fld (fld_WPA_THRESHOLD,    RT_NTSCM_WPA_THRESHOLD);

WriteRT_fld (fld_WPA_TRIGGER_LO,   RT_NTSCM_WPA_TRIGGER_LO);

WriteRT_fld (fld_WPA_TRIGGER_HIGH, RT_NTSCM_WPA_TRIGGER_HIGH);

WriteRT_fld (fld_LOCKOUT_START,    RT_NTSCM_LP_LOCKOUT_START);

WriteRT_fld (fld_LOCKOUT_END,      RT_NTSCM_LP_LOCKOUT_END);

// Program the Chroma demodulator

WriteRT_fld (fld_CH_DTO_INC,       RT_NTSCM_CH_DTO_INC);

WriteRT_fld (fld_PLL_SGAIN,        RT_NTSCM_CH_PLL_SGAIN);

WriteRT_fld (fld_PLL_FGAIN,        RT_NTSCM_CH_PLL_FGAIN);

WriteRT_fld (fld_CH_HEIGHT,        RT_NTSCM_CH_HEIGHT);

WriteRT_fld (fld_CH_KILL_LEVEL,    RT_NTSCM_CH_KILL_LEVEL);

WriteRT_fld (fld_CH_AGC_ERROR_LIM, RT_NTSCM_CH_AGC_ERROR_LIM);

WriteRT_fld (fld_CH_AGC_FILTER_EN, RT_NTSCM_CH_AGC_FILTER_EN);

WriteRT_fld (fld_CH_AGC_LOOP_SPEED,RT_NTSCM_CH_AGC_LOOP_SPEED);

WriteRT_fld (fld_CR_BURST_GAIN,        RT_NTSCM_CR_BURST_GAIN);

WriteRT_fld (fld_CB_BURST_GAIN,        RT_NTSCM_CB_BURST_GAIN);

WriteRT_fld (fld_CRDR_ACTIVE_GAIN,     RT_NTSCM_CRDR_ACTIVE_GAIN);

WriteRT_fld (fld_CBDB_ACTIVE_GAIN,     RT_NTSCM_CBDB_ACTIVE_GAIN);

WriteRT_fld (fld_VERT_LOCKOUT_START,   RT_NTSCM_VERT_LOCKOUT_START);

WriteRT_fld (fld_VERT_LOCKOUT_END,     RT_NTSCM_VERT_LOCKOUT_END);

// Program the VSYNC PLL

WriteRT_fld (fld_VSYNC_INT_TRIGGER, (WORD)RT_NTSCM_VSYNC_INT_TRIGGER);

WriteRT_fld (fld_VSYNC_INT_HOLD, (WORD)((dbLPeriod * dbFsamp / 200.0)

+ 0.5 + 20));

WriteRT_fld (fld_VS_FIELD_BLANK_START, RT_NTSCM_VS_FIELD_BLANK_START);

WriteRT_fld (fld_VS_FIELD_BLANK_END,   RT_NTSCM_VS_FIELD_BLANK_END);

WriteRT_fld (fld_VS_FIELD_IDLOCATION, (WORD)(wFrameTotal / 2.0 - 1.5));

WriteRT_fld (fld_VS_FRAME_TOTAL, (WORD)(wFrameTotal) + 10);

// Program the clipping engine

WriteRT_fld (fld_H_ACTIVE_START,   RT_NTSCM_H_ACTIVE_START);

WriteRT_fld (fld_H_ACTIVE_END,     RT_NTSCM_H_ACTIVE_END);

WriteRT_fld (fld_V_ACTIVE_START,   RT_NTSCM_V_ACTIVE_START);

WriteRT_fld (fld_V_ACTIVE_END,     RT_NTSCM_V_ACTIVE_END);

WriteRT_fld (fld_H_VBI_WIND_START, RT_NTSCM_H_VBI_WIND_START);

WriteRT_fld (fld_H_VBI_WIND_END,   RT_NTSCM_H_VBI_WIND_END);

WriteRT_fld (fld_V_VBI_WIND_START, RT_NTSCM_V_VBI_WIND_START);

WriteRT_fld (fld_V_VBI_WIND_END,   RT_NTSCM_V_VBI_WIND_END);

// Program the comb filter

if (RT_INFO->wConnector == DEC_SVIDEO)

{

// Sets up the input comb filter, based on the video standard, and

// the type of video input (composite vs. svideo) selected.

RT_SetCombFilter (wStandard, RT_SVIDEO);

}

else

{

RT_SetCombFilter (wStandard, RT_COMPOSITE);

}

// Program the HSYNC PLL

WriteRT_fld (fld_HS_LINE_TOTAL, (WORD)((dbLPeriod*dbFsamp / 2.0) + 0.5));

WriteRT_fld (fld_MIN_PULSE_WIDTH, (BYTE)0x32);

WriteRT_fld (fld_MAX_PULSE_WIDTH, (BYTE)0x64);

WriteRT_fld (fld_WIN_OPEN_LIMIT, (WORD)((dbLPeriod * dbFsamp / 4.0) +

0.5) - 16);

// Program the Timing Generator

WriteRT_fld (fld_UV_INT_START, (BYTE)(((0.10 * dbLPeriod * dbFsamp / 2.0)

+ 0.5) - 32));

WriteRT_fld (fld_BLACK_INT_START, (BYTE)((0.09 * dbLPeriod * dbFsamp /

2.0) - 32));

WriteRT_fld (fld_SYNC_TIP_START, (WORD)((dbLPeriod * dbFsamp / 2.0 +

0.5) - 28));

return;

} // RT_SetStandard ()...

## 1.3 Setting the DVS port for video output

The Rage Theatre uses the DVS port to send the decoded video back to the graphics chip.  To enable the DVS port, and configure it for output, set DVS_DIRECTION @ DVS_PORT_CTRL to 1 for output.  When setting this bit, make sure that no other bitfields are modified,  unless required.

## 1.4 enabling the analog to digital convertor

The analog to digital convertor (ADC) used by the Rage Theatre must now be configured.  This is done by setting the register ADC_CNTL.  This register controls such things as the actual physical input selected for the Rage Theatre, the input decimator and related filtering, and the overall behaviour of the ADC.

The first field that is set to indicate the type of connector being used is the INPUT_SELECT field.  The following table shows the possible values for INPUT_SELECT, and what physical connector is selected based on the value of this field.

| INPUT_SELECT | Function |
| --- | --- |
| 0* | Select Comp0 input (composite video mode) |
| 1 | Select Comp1 input (composite video mode) |
| 2 | Select Comp2 input (composite video mode) |
| 3 | Select YF_Comp3 input (composite video mode) |
| 4 | Select YR_Comp4 input (composite video mode) |
| 5 | Select YF_Comp3, CF inputs (YC video mode) |
| 6 | Select YR_Comp3, CR inputs (YC video mode) |

*The default for the INPUT_SELECT field is 0.

The rest of the fields must be set up, and the following table shows the values that should be used:

ADC_CNTL

| Field | All modes |
| --- | --- |
| I_CLAMP_SEL [4:3] | 3 |
| I_AGC_SEL [6:5] | 1 |
| *ADC_PDWN [7] | 0 |
| EXT_CLAMP_CAP [8] | 1 |
| EXT_AGC_CAP [9] | 1 |
| ADC_DECI_BYPASS [10] | 0 |
| VBI_DECI_BYPASS [11] | 0 |
| DECI_DITHER_EN [12] | 0 |
| ADC_CLK_SEL [13] | 1 |
| ADC_BYPASS [15:14] | 0 |
| ADC_CH_GAIN_SEL [17:16] | 1 |
| ADC_PAICM [19:18] | 1 |
| ADC_PDCBIAS [21:20] | 2 |
| ADC_PREFHI [23:22] | 0 |
| ADC_PREFLO [25:24] | 3 |
| ADC_IMUXOFF [26] | 0 |
| ADC_CPRESET [27] | 0 |

*The default for the ADC_RDWN field is 1, this field must be set to 0 to capture video.

## 1.5 enabling the video in, scaler and dvs port blocks

At this point, the Rage Theatre is fully initialized.  The only remaining task is to enable the video in and DVS port blocks.  This is done by simply clearing VIN_ASYNC_RST @ MASTER_CNTL and DVS_ASYNC_RST @ MASTER_CNTL

## 1.6 SUMMARIZING THE STEPS – ENABLING THE RAGE THEATRE DECODER

Sections 1.1 through 1.5 detail the steps necessary to initialize and enable video decoding with the Rage Theatre.  The following code puts those steps into practice:

void RT_EnableDecoder (void)

{

DWORD     data;

// 1.

// Set the VIN_PLL to NTSC value

RT_SetVINClock (RT_NTSC);

// Take VINRST and L54RST out of reset

RT_regr (VIP_PLL_CNTL1, &data);

RT_regw (VIP_PLL_CNTL1, data & ~((RT_VINRST_RESET << 1) |

(RT_L54RST_RESET << 3)));

RT_regr (VIP_PLL_CNTL1, &data);

// Set VIN_CLK_SEL to PLL_VIN_CLK

RT_regr (VIP_CLOCK_SEL_CNTL, &data);

RT_regw (VIP_CLOCK_SEL_CNTL, data | (RT_PLL_VIN_CLK << 7));

RT_regr (VIP_CLOCK_SEL_CNTL, &data);

// Clear the VIN_ASYNC_RST bit

RT_regr (VIP_MASTER_CNTL, &data);

RT_regw (VIP_MASTER_CNTL, data & ~(RT_VIN_ASYNC_RST));

RT_regr (VIP_MASTER_CNTL, &data);

// 2.

// Set HW_DEBUG to 0xF000 before setting the

// standards registers

RT_regw (VIP_HW_DEBUG, 0x0000F000);

RT_SetStandard (RT_INFO->wStandard);

// 3.

// Set DVS port to OUTPUT

RT_regr (VIP_DVS_PORT_CTRL, &data);

RT_regw (VIP_DVS_PORT_CTRL, data | RT_DVSDIR_OUT);

RT_regr (VIP_DVS_PORT_CTRL, &data);

// 4.

// Set default values for ADC_CNTL

RT_regw (VIP_ADC_CNTL, RT_ADC_CNTL_DEFAULT);

// Reset the VINPLL

// Set VIN_CLK_SEL to REF_CLK

RT_regr (VIP_CLOCK_SEL_CNTL, &data);

RT_regw (VIP_CLOCK_SEL_CNTL, data & ~0x80);

// Put VIN into reset

RT_regr (VIP_PLL_CNTL1, &data);

RT_regw (VIP_PLL_CNTL1, data | 0x2);

// Program VIN_PLL_CNTL for NTSC

RT_SetVINClock (RT_NTSC);

// Take VIN out of reset

RT_regr (VIP_PLL_CNTL1, &data);

RT_regw (VIP_PLL_CNTL1, data & ~0x2);

// Set VIN_CLK_SEL to PLL_VIN_CLK

RT_regr (VIP_CLOCK_SEL_CNTL, &data);

RT_regw (VIP_CLOCK_SEL_CNTL, data | 0x80);

// 5.

// Clear the VIN_ASYNC_RST bit

RT_regr (VIP_MASTER_CNTL, &data);

RT_regw (VIP_MASTER_CNTL, data & ~0x20);

RT_regr (VIP_MASTER_CNTL, &data);

// Clear the DVS_ASYNC_RST bit

RT_regr (VIP_MASTER_CNTL, &data);

RT_regw (VIP_MASTER_CNTL, data & ~(RT_DVS_ASYNC_RST));

RT_regr (VIP_MASTER_CNTL, &data);

RT_regw (VIP_HS_GENLOCKDELAY, 0x10);

} // RT_EnableDecoder ()...

# 2.0 video quality settings

As with a standard television, there are some basic settings that affect the video image: brightness, contrast, saturation, and tint (hue). The Rage Theatre allows all of these settings to be controlled via registers.  While modifying the registers themselves is a simple task, there are formulas that determine the actual value for these settings for a given video standard.

## 2.1 Brightness

Brightness affects the intensity of the video signal, and refers to how much light is emitted by the display.  For the Rage Theatre, the brightness level is controlled by the register LP_BRIGHTNESS.  The following code demonstrates how to set the brightness level of the video decoder in the Rage Theatre.  The integer passed to this function should be in the range of 0 to 200, with 100 being a good default value.

void RT_SetBrightness (int Brightness)

{

double dbSynctipRef0=0, dbContrast=1;

double dbYgain=0;

double dbBrightness=0;

double dbSetup=0;

WORD   wBrightness=0;

// VALIDATE BRIGHTNESS LEVEL

if (Brightness < 0)

{

Brightness = 0;

}

else if (Brightness > 200)

{

Brightness = 200;

}

// Save value

RT_INFO->iBrightness = Brightness;

RT_INFO->dbBrightnessRatio =  (double) Brightness / 100.0;

dbBrightness = (double) (Brightness - 100);

dbSynctipRef0 = ReadRT_fld (fld_SYNCTIP_REF0);

if(RT_INFO->dbContrast == 0)

{

RT_INFO->dbContrast = 1.0; //NTSC default;

}

dbContrast = (double) RT_INFO->dbContrast;

// Use the following formula to calculate the brightness value

switch (RT_INFO->wStandard & 0x00FF)

{

case (DEC_NTSC):

if( (RT_INFO->wStandard & 0xFF00) == extNTSC_J)

dbYgain = 219.0 / ( 100.0 * (double)(dbSynctipRef0) /40.0);

else

{

dbSetup = 7.5 * (double)(dbSynctipRef0) /40.0;

dbYgain = 219.0 / ( 92.5 * (double)(dbSynctipRef0) /40.0);

}

break;

case (DEC_PAL):

case (DEC_SECAM):

dbYgain = 219.0 / ( 100.0 * (double)(dbSynctipRef0) /43.0);

break;

default:

break;

}

wBrightness = (WORD) (16.0 * ((dbBrightness-dbSetup) + (16.0 /

(dbContrast * dbYgain))));

WriteRT_fld (fld_LP_BRIGHTNESS, wBrightness);

return;

} // RT_SetBrightness ()...

## 2.2 Contrast

Contrast, in video terms, is the difference between white and black peak levels.  An image that has a high contrast value will have very distinct black and white levels, while low contrast images tend to be washed out.  The register LP_CONTRAST determines the contrast level of the video decoder.  The following code demonstrates the formula used to determine the contrast setting.  Once again, the input should be between 0 and 200, with 100 being a good default.

void RT_SetContrast (int Contrast)

{

double dbSynctipRef0=0, dbContrast=0;

double dbYgain=0;

BYTE   bTempContrast=0;

// VALIDATE CONTRAST LEVEL

if (Contrast < 0)

{

Contrast = 0;

}

else if (Contrast > 200)

{

Contrast = 200;

}

// Save contrast value

RT_INFO->iContrast = Contrast;

dbSynctipRef0 = ReadRT_fld (fld_SYNCTIP_REF0);

dbContrast = (double) Contrast / 100.0;

switch (RT_INFO->wStandard & 0x00FF)

{

case (DEC_NTSC):

if ((RT_INFO->wStandard & 0xFF00) == (extNTSC_J))

{

dbYgain = 219.0 / ( 100.0 * (double)(dbSynctipRef0) /40.0);

}

else

{

dbYgain = 219.0 / ( 92.5 * (double)(dbSynctipRef0) /40.0);

}

break;

case (DEC_PAL):

case (DEC_SECAM):

dbYgain = 219.0 / ( 100.0 * (double)(dbSynctipRef0) /43.0);

break;

default:

break;

}

bTempContrast = (BYTE) ((dbContrast * dbYgain * 64) + 0.5);

WriteRT_fld (fld_LP_CONTRAST, (DWORD)bTempContrast);

// Save value for future modification

RT_INFO->dbContrast = dbContrast;

return;

} // RT_SetContrast ()...

## 2.3 Saturation

Saturation refers to the amount of pigment in an individual colour.  Basically, the more white mixed in with a particular colour, the lower the saturation.  For example, a lightly saturated red would look more pink, where a highly saturated red would be more fire-engine red.  The following code sets the saturation of the video decoder for the Rage Theatre. Once again, the input should be between 0 and 200, with 100 being a good default.

void RT_SetSaturation (int Saturation)

{

WORD   wSaturation_V, wSaturation_U;

double dbSaturation = 0, dbCrGain = 0, dbCbGain = 0;

// VALIDATE SATURATION LEVEL

if (Saturation < 0L)

{

Saturation = 0;

}

else if (Saturation > 200L)

{

Saturation = 200;

}

RT_INFO->iSaturation = Saturation;

dbSaturation = (double) Saturation / 100.0;

CalculateCrCbGain (&dbCrGain, &dbCbGain, RT_INFO->wStandard);

wSaturation_U = (WORD) ((dbCrGain * dbSaturation * 128.0) + 0.5);

wSaturation_V = (WORD) ((dbCbGain * dbSaturation * 128.0) + 0.5);

// SET SATURATION LEVEL

WriteRT_fld (fld_CRDR_ACTIVE_GAIN, wSaturation_U);

WriteRT_fld (fld_CBDB_ACTIVE_GAIN, wSaturation_V);

RT_INFO->wSaturation_U = wSaturation_U;

RT_INFO->wSaturation_V = wSaturation_V;

return;

} // RT_SetSaturation ()...

## 2.4 Tint/Hue

Tint or hue, determines the wavelength of colour.  It is separate from the saturation or brightness of a colour, in that it affects the baseline of the colour itself.   The following code accepts an input from -180 to 180, with 0 being a good default value.

void RT_SetTint (int hue)

{

DWORD nhue;

// Validate Hue level

if (hue < -180)

{

hue = -180;

}

else if (hue > 180)

{

hue = 180;

}

RT_INFO->iHue = hue;

switch (RT_INFO->wStandard & 0x00FF)

{

case (DEC_NTSC):

if (hue >= 0)

{

nhue = (DWORD) (256 * hue)/360;

}

else

{

nhue = (DWORD) (256 * (hue + 360))/360;

}

break;

case (DEC_PAL):

case (DEC_SECAM):

break;

default:            break;

}

WriteRT_fld (fld_CP_HUE_CNTL, nhue);

return;

} // RT_SetTint ()...

# 3.0 Vertical blank interval data

The Rage Theatre is fully capable of determining the amount and size of vertical blank data that can be transmitted to the capture device.  The amount of lines transferred and the width of the VBI line can be set by the V_VBI_WINDOW and H_VBI_WINDOW registers respectively.  VBI_CONTROL enables VBI capture.  The following tables show values that can be used for these registers, depending on the standard.

H_VBI_WINDOW

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H_VBI_WIND_START | 0x064 | 0x064 | 0x064 | 0x064 | 0x084 | 0x084 | 0x084 | 0x064 | 0x084 |
| H_VBI_WIND_END | 0x366 | 0x366 | 0x366 | 0x366 | 0x41F | 0x41F | 0x41F | 0x366 | 0x41F |

V_VBI_WINDOW

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V_VBI_WIND_START | 0x00b | 0x00b | 0x00b | 0x00b | 0x008 | 0x008 | 0x008 | 0x00b | 0x008 |
| V_VBI_WIND_END | 0x024 | 0x024 | 0x024 | 0x024 | 0x02d | 0x02d | 0x02d | 0x024 | 0x02d |

VBI_CONTROL

| VBI_CAPTURE_EN ABLE | Mode |
| --- | --- |
| 0 | Disable VBI Capture |
| 1 | Enable VBI Capture |
| 2 | Enable Raw Video Capture |

Once the data has been captured by the host device, it must be processed to provide the proper data.  To properly process the data, consult the specification of the VBI standard being implemented (i.e. Closed Captioning, Teletext, etc.).

# 4.0 Comb filter settings

The input comb filter must be programmed according to the selected standard.  The tables following show the values that should be used for the appropriate comb filter registers.

## 4.1 NTSC-M, NTSC-Japan Comb Filter Modes

COMB_CNTL0 Comb filter control register 0

| COMB_CNTL0 | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | 2Tap C-combed Y-combed | 3Tap C-combed, Y-Sub | 3Tap C-combed, Y-combed | YC mode, Comb filter off | YC mode, 2Tap YV filter | YC mode, 3Tap YV filter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| COMB_HCK | 0x90 | X | X | X | X | X | X | X |
| COMB_VCK | 0x80 | X | X | X | X | X | X | X |
| COMB_FILTER_EN | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| COMB_ADAPTIVE_EN | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| COMB_BPFMUXSEL | 0 | 3 | 3 | 1 | 1 | 5 | 5 | 5 |
| COMB_COUTSEL | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| COMB_SUMDIFF0SEL | 0 | 0 | 0 | 0 | 0 | X | 0 | X |
| COMB_SUMDIFF1SEL | 1 | 0 | 0 | 1 | 1 | X | 0 | X |
| COMB_YVLPFSEL | 0 | 0 | 0 | 0 | 1 | X | 0 | 1 |
| COMB_DLYLINESEL | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| COMB_YDLYINSEL | 0 | 0 | 1 | 0 | 0 | 2 | 1 | 0 |
| COMB_YSUBBW | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

COMB_CNTL1 Comb filter control register 1

| COMB_CNTL1 | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | 2Ta C-combed Y-combed | 3Tap C-combed, Y-Sub | 3Tap C-combed, Y-combed | YC mode, Comb filter off | YC mode, 2Tap YV filter | YC mode, 2Tap YV filter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| COMB_YDLYOUTSEL | 0 | 2 | 3 | 0 | 3 | 1 | 3 | 3 |
| COMB_CORESIZE | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| COMB_YSUBEN | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| COMB_YOUTSEL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| COMB_SYNCLPFSEL | 0 | 0 | 0 | 1 | 1 | 2 | 0 | 1 |
| COMB_SYNCLPFRST | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| COMB_DEBUG | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

COMB_CNTL2 Comb filter control register 2

| COMB_CNTL2 | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | 2Ta C-combed Y-combed | 3Tap C-combed, Y-Sub | 3Tap C-combed, Y-combed | YC mode, Comb filter off | YC mode, 2Tap YV filter | YC mode, 2Tap YV filter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| COMB_HYK0 | 0x10 | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF |
| COMB_VYK0 | 0x10 | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF |
| COMB_HYK1 | 0x16 | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF |
| COMB_VYK1 | 0x16 | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF |

COMB_LINE_LENGTH Comb filter line length register

| COMB_LINE_LENGTH | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | 2Ta C-combed Y-combed | 3Tap C-combed, Y-Sub | 3Tap C-combed, Y-combed | YC mode, Comb filter off | YC mode, 2Tap YV filter | YC mode, 2Tap YV filter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| COMB_TAP0LENGTH | 0x38A | 0x38A | 0x38A | 0x38A | 0x38A | X | 0x38A | 0x38A |
| COMB_TAP1LENGTH | 0x718 | 0x718 | 0x718 | 0x718 | 0x718 | X | X | 0x718 |

## 4.2 NTSC-443 Comb Filter Modes

COMB_CNTL0 Comb filter control register 0

| COMB_CNTL0 | 2Tap C-combed Y-Sub. | 2Tap C-combed Y-combed | 3Tap C-combed, Y-Sub | 3Tap C-combed, Y-combed | YC mode, Comb filter off | YC mode, 2Tap YV filter | YC mode, 3Tap YV filter |
| --- | --- | --- | --- | --- | --- | --- | --- |
| COMB_HCK | X | X | X | X | X | X | X |
| COMB_VCK | X | X | X | X | X | X | X |
| COMB_FILTER_EN | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| COMB_ADAPTIVE_EN | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| COMB_BPFMUXSEL | 3 | 3 | 1 | 1 | 5 | 5 | 5 |
| COMB_COUTSEL | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| COMB_SUMDIFF0SEL | 0 | 0 | 0 | 0 | X | 0 | X |
| COMB_SUMDIFF1SEL | 0 | 0 | 1 | 1 | X | 0 | X |
| COMB_YVLPFSEL | 0 | 0 | 0 | 1 | X | 0 | 1 |
| COMB_DLYLINESEL | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| COMB_YDLYINSEL | 0 | 1 | 0 | 0 | 2 | 1 | 0 |
| COMB_YSUBBW | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

COMB_CNTL1 Comb filter control register 1

| COMB_CNTL1 | 2Tap C-combed Y-Sub. | 2Ta C-combed Y-combed | 3Tap C-combed, Y-Sub | 3Tap C-combed, Y-combed | YC mode, Comb filter off | YC mode, 2Tap YV filter | YC mode, 2Tap YV filter |
| --- | --- | --- | --- | --- | --- | --- | --- |
| COMB_YDLYOUTSEL | 2 | 3 | 0 | 3 | 1 | 3 | 3 |
| COMB_CORESIZE | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| COMB_YSUBEN | 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| COMB_YOUTSEL | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| COMB_SYNCLPFSEL | 0 | 0 | 1 | 1 | 2 | 0 | 1 |
| COMB_SYNCLPFRST | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| COMB_DEBUG | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

COMB_CNTL2 Comb filter control register 2

| COMB_CNTL2 | 2Tap C-combed Y-Sub. | 2Ta C-combed Y-combed | 3Tap C-combed, Y-Sub | 3Tap C-combed, Y-combed | YC mode, Comb filter off | YC mode, 2Tap YV filter | YC mode, 2Tap YV filter |
| --- | --- | --- | --- | --- | --- | --- | --- |
| COMB_HYK0 | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF |
| COMB_VYK0 | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF |
| COMB_HYK1 | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF |
| COMB_VYK1 | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF |

COMB_LINE_LENGTH Comb filter line length register

| COMB_LINE_LENGTH | 2Tap C-combed Y-Sub. | 2Ta C-combed Y-combed | 3Tap C-combed, Y-Sub | 3Tap C-combed, Y-combed | YC mode, Comb filter off | YC mode, 2Tap YV filter | YC mode, 2Tap YV filter |
| --- | --- | --- | --- | --- | --- | --- | --- |
| COMB_TAP0LENGTH | 0x462 | 0x462 | 0x462 | 0x462 | X | 0x462 | 0x462 |
| COMB_TAP1LENGTH | 0x8C9 | 0x8C9 | 0x8C9 | 0x8C9 | X | 0x8C9 | 0x8C9 |

## 4.3 PAL-M Comb Filter Modes

COMB_CNTL0 Comb filter control register 0

| COMB_CNTL0 | 2Tap C-combed Y-Sub. | YC mode, 2Tap C-combed |
| --- | --- | --- |
| COMB_HCK | X | X |
| COMB_VCK | X | X |
| COMB_FILTER_EN | 1 | 1 |
| COMB_ADAPTIVE_EN | 0 | 0 |
| COMB_BPFMUXSEL | 4 | 5 |
| COMB_COUTSEL | 0 | 0 |
| COMB_SUMDIFF0SEL | 1 | 1 |
| COMB_SUMDIFF1SEL | 2 | 2 |
| COMB_YVLPFSEL | 0 | 0 |
| COMB_DLYLINESEL | 0 | 0 |
| COMB_YDLYINSEL | 2 | 2 |
| COMB_YSUBBW | 0 | 0 |

COMB_CNTL1 Comb filter control register 1

| COMB_CNTL1 | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- |
| COMB_YDLYOUTSEL | 1 | 1 |
| COMB_CORESIZE | 0 | 0 |
| COMB_YSUBEN | 1 | 0 |
| COMB_YOUTSEL | 1 | 1 |
| COMB_SYNCLPFSEL | 2 | 2 |
| COMB_SYNCLPFRST | 0 | 0 |
| COMB_DEBUG | 0 | 0 |

COMB_CNTL2 Comb filter control register 2

| COMB_CNTL2 | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- |
| COMB_HYK0 | 0xFF | 0xFF |
| COMB_VYK0 | 0xFF | 0xFF |
| COMB_HYK1 | 0xFF | 0xFF |
| COMB_VYK1 | 0xFF | 0xFF |

COMB_LINE_LENGTH Comb filter line length register

| COMB_LINE_LENGTH | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- |
| COMB_TAP0LENGTH | 0x389 | 0x389 |
| COMB_TAP1LENGTH | X | X |

## 4.4 PAL-N Comb Filter Modes

COMB_CNTL0 Comb filter control register 0

| COMB_CNTL0 | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | YC mode, 2Tap C-combed |
| --- | --- | --- | --- |
| COMB_HCK | 0x90 | X | X |
| COMB_VCK | 0x80 | X | X |
| COMB_FILTER_EN | 1 | 1 | 1 |
| COMB_ADAPTIVE_EN | 1 | 0 | 0 |
| COMB_BPFMUXSEL | 0 | 4 | 5 |
| COMB_COUTSEL | 2 | 0 | 0 |
| COMB_SUMDIFF0SEL | 0 | 1 | 1 |
| COMB_SUMDIFF1SEL | 1 | 2 | 2 |
| COMB_YVLPFSEL | 0 | 0 | X |
| COMB_DLYLINESEL | 1 | 0 | 0 |
| COMB_YDLYINSEL | 0 | 2 | 2 |
| COMB_YSUBBW | 0 | 0 | 0 |

COMB_CNTL1 Comb filter control register 1

| COMB_CNTL1 | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- | --- |
| COMB_YDLYOUTSEL | 0 | 1 | 1 |
| COMB_CORESIZE | 0 | 0 | 0 |
| COMB_YSUBEN | 1 | 1 | 0 |
| COMB_YOUTSEL | 0 | 1 | 1 |
| COMB_SYNCLPFSEL | 0 | 2 | 2 |
| COMB_SYNCLPFRST | 0 | 0 | 0 |
| COMB_DEBUG | 0 | 0 | 0 |

COMB_CNTL2 Comb filter control register 2

| COMB_CNTL2 | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- | --- |
| COMB_HYK0 | 0x10 | 0xFF | 0xFF |
| COMB_VYK0 | 0x10 | 0xFF | 0xFF |
| COMB_HYK1 | 0x16 | 0xFF | 0xFF |
| COMB_VYK1 | 0x16 | 0xFF | 0xFF |

COMB_LINE_LENGTH Comb filter line length register

| COMB_LINE_LENGTH | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- | --- |
| COMB_TAP0LENGTH | 0x46B | 0x46C | 0x46C |
| COMB_TAP1LENGTH | 0x8DA | X | X |

## 4.5 PAL-N combination Comb Filter Modes

COMB_CNTL0 Comb filter control register 0

| COMB_CNTL0 | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | YC mode, 2Tap C-combed |
| --- | --- | --- | --- |
| COMB_HCK | 0x90 | X | X |
| COMB_VCK | 0x80 | X | X |
| COMB_FILTER_EN | 1 | 1 | 1 |
| COMB_ADAPTIVE_EN | 1 | 0 | 0 |
| COMB_BPFMUXSEL | 0 | 4 | 5 |
| COMB_COUTSEL | 2 | 0 | 0 |
| COMB_SUMDIFF0SEL | 0 | 1 | 1 |
| COMB_SUMDIFF1SEL | 1 | 2 | 2 |
| COMB_YVLPFSEL | 0 | 0 | X |
| COMB_DLYLINESEL | 1 | 0 | 0 |
| COMB_YDLYINSEL | 0 | 2 | 2 |
| COMB_YSUBBW | 0 | 0 | 0 |

COMB_CNTL1 Comb filter control register 1

| COMB_CNTL1 | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- | --- |
| COMB_YDLYOUTSEL | 0 | 1 | 1 |
| COMB_CORESIZE | 0 | 0 | 0 |
| COMB_YSUBEN | 1 | 1 | 0 |
| COMB_YOUTSEL | 0 | 1 | 1 |
| COMB_SYNCLPFSEL | 0 | 2 | 2 |
| COMB_SYNCLPFRST | 0 | 0 | 0 |
| COMB_DEBUG | 0 | 0 | 0 |

COMB_CNTL2 Comb filter control register 2

| COMB_CNTL2 | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- | --- |
| COMB_HYK0 | 0x10 | 0xFF | 0xFF |
| COMB_VYK0 | 0x10 | 0xFF | 0xFF |
| COMB_HYK1 | 0x16 | 0xFF | 0xFF |
| COMB_VYK1 | 0x16 | 0xFF | 0xFF |

COMB_LINE_LENGTH Comb filter line length register

| COMB_LINE_LENGTH | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- | --- |
| COMB_TAP0LENGTH | 0x391 | 0x394 | 0x394 |
| COMB_TAP1LENGTH | 0x726 | X | X |

## 4.6 PAL-BDGHI Comb Filter Modes

COMB_CNTL0 Comb filter control register 0

| COMB_CNTL0 | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | YC mode, 2Tap C-combed |
| --- | --- | --- | --- |
| COMB_HCK | 0x90 | X | X |
| COMB_VCK | 0x80 | X | X |
| COMB_FILTER_EN | 1 | 1 | 1 |
| COMB_ADAPTIVE_EN | 1 | 0 | 0 |
| COMB_BPFMUXSEL | 0 | 4 | 5 |
| COMB_COUTSEL | 2 | 0 | 0 |
| COMB_SUMDIFF0SEL | 0 | 1 | 1 |
| COMB_SUMDIFF1SEL | 1 | 2 | 2 |
| COMB_YVLPFSEL | 0 | 0 | X |
| COMB_DLYLINESEL | 1 | 0 | 0 |
| COMB_YDLYINSEL | 0 | 2 | 2 |
| COMB_YSUBBW | 0 | 0 | 0 |

COMB_CNTL1 Comb filter control register 1

| COMB_CNTL1 | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- | --- |
| COMB_YDLYOUTSEL | 0 | 1 | 1 |
| COMB_CORESIZE | 0 | 0 | 0 |
| COMB_YSUBEN | 1 | 1 | 0 |
| COMB_YOUTSEL | 0 | 1 | 1 |
| COMB_SYNCLPFSEL | 0 | 2 | 2 |
| COMB_SYNCLPFRST | 0 | 0 | 0 |
| COMB_DEBUG | 0 | 0 | 0 |

COMB_CNTL2 Comb filter control register 2

| COMB_CNTL2 | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- | --- |
| COMB_HYK0 | 2 | 0xFF | 0xFF |
| COMB_VYK0 | 1 | 0xFF | 0xFF |
| COMB_HYK1 | 8 | 0xFF | 0xFF |
| COMB_VYK1 | 6 | 0xFF | 0xFF |

COMB_LINE_LENGTH Comb filter line length register

| COMB_LINE_LENGTH | 3Tap 2D adaptive Comb | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- | --- |
| COMB_TAP0LENGTH | 0x46B | 0x46C | 0x46C |
| COMB_TAP1LENGTH | 0x8DA | X | X |

## 4.7 PAL-60 Comb Filter Modes

COMB_CNTL0 Comb filter control register 0

| COMB_CNTL0 | 2Tap C-combed Y-Sub. | YC mode, 2Tap C-combed |
| --- | --- | --- |
| COMB_HCK | X | X |
| COMB_VCK | X | X |
| COMB_FILTER_EN | 1 | 1 |
| COMB_ADAPTIVE_EN | 0 | 0 |
| COMB_BPFMUXSEL | 4 | 5 |
| COMB_COUTSEL | 0 | 0 |
| COMB_SUMDIFF0SEL | 1 | 1 |
| COMB_SUMDIFF1SEL | 2 | 2 |
| COMB_YVLPFSEL | 0 | 0 |
| COMB_DLYLINESEL | 0 | 0 |
| COMB_YDLYINSEL | 2 | 2 |
| COMB_YSUBBW | 0 | 0 |

COMB_CNTL1 Comb filter control register 1

| COMB_CNTL1 | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- |
| COMB_YDLYOUTSEL | 1 | 1 |
| COMB_CORESIZE | 0 | 0 |
| COMB_YSUBEN | 1 | 0 |
| COMB_YOUTSEL | 1 | 1 |
| COMB_SYNCLPFSEL | 2 | 2 |
| COMB_SYNCLPFRST | 0 | 0 |
| COMB_DEBUG | 0 | 0 |

COMB_CNTL2 Comb filter control register 2

| COMB_CNTL2 | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- |
| COMB_HYK0 | 0xFF | 0xFF |
| COMB_VYK0 | 0xFF | 0xFF |
| COMB_HYK1 | 0xFF | 0xFF |
| COMB_VYK1 | 0xFF | 0xFF |

COMB_LINE_LENGTH Comb filter line length register

| COMB_LINE_LENGTH | 2Tap C-combed Y-Sub. | YC mode, 2Tap YV filter |
| --- | --- | --- |
| COMB_TAP0LENGTH | 0x463 | 0x463 |
| COMB_TAP1LENGTH | X | X |

## 4.8 SECAM-BDGHKL Comb Filter Modes

COMB_CNTL0 Comb filter control register 0

| COMB_CNTL0 | Composite Mode | YC Mode |
| --- | --- | --- |
| COMB_HCK | X | X |
| COMB_VCK | X | X |
| COMB_FILTER_EN | 0 | 0 |
| COMB_ADAPTIVE_EN | 0 | 0 |
| COMB_BPFMUXSEL | 4 | 5 |
| COMB_COUTSEL | X | X |
| COMB_SUMDIFF0SEL | X | X |
| COMB_SUMDIFF1SEL | X | X |
| COMB_YVLPFSEL | X | X |
| COMB_DLYLINESEL | 2 | 2 |
| COMB_YDLYINSEL | 2 | 2 |
| COMB_YSUBBW | 1 | X |

COMB_CNTL1 Comb filter control register 1

| COMB_CNTL1 | Composite Mode | YC Mode |
| --- | --- | --- |
| COMB_YDLYOUTSEL | 1 | 1 |
| COMB_CORESIZE | 0 | X |
| COMB_YSUBEN | 1 | 0 |
| COMB_YOUTSEL | 0 | 0 |
| COMB_SYNCLPFSEL | 2 | 2 |
| COMB_SYNCLPFRST | 0 | 0 |
| COMB_DEBUG | 0 | 0 |

COMB_CNTL2 Comb filter control register 2

| COMB_CNTL1 | Composite Mode | YC Mode |
| --- | --- | --- |
| COMB_HYK0 | 0xFF | 0xFF |
| COMB_VYK0 | 0xFF | 0xFF |
| COMB_HYK1 | 0xFF | 0xFF |
| COMB_VYK1 | 0xFF | 0xFF |

COMB_LINE_LENGTH Comb filter line length register

| COMB_LINE_LENGTH | Composite Mode | YC Mode |
| --- | --- | --- |
| COMB_TAP0LENGTH | 0x46A | 0x46A |
| COMB_TAP1LENGTH | X | X |

# 5.0 Register Tables

This section contains tables of values for Rage Theatre registers, as used in the RT_SetStandard () function.  The values are shown for each available video standard.

## 5.1 Clamp Control

LP_AGC_CLAMP_CNTL0 AGC/Clamp control register 0

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- GBHIDKK1 | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SYNCTIP_REF0 | 0x037 | 0x037 | 0x037 | 0x037 | 0x037 | 0x037 | 0x037 | 0x037 | 0x037 |
| SYNVTIP_REF1 | 0x029 | 0x029 | 0x029 | 0x029 | 0x029 | 0x026 | 0x026 | 0x026 | 0x026 |
| CLAMP_REF | 0x03B | 0x03B | 0x03B | 0x03B | 0x03B | 0x03B | 0x03B | 0x03B | 0x03B |
| AGC_PEAKWHITE | 0x0FF | 0x0FF | 0x0FF | 0x0FF | 0x0FF | 0x0FF | 0x0FF | 0x0FF | 0x0FF |

LP_AGC_CLAMP_CNTL1 AGC/Clamp control register 1

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VBI_PEAKWHITE | 0x0D2 | 0x0D2 | 0xD2 | 0x0D2 | 0x0D2 | 0x0C6 | 0x0C6 | 0x0C6 | 0x0C6 |

## 5.2 Luma Processor

LP_WPA_CNTL0 White peak accumulation control register 0

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WPA_THRESHOLD | 0x406 | 0x406 | 0x4FC | 0x406 | 0x59C | 0x488 | 0x59C | 0x59C | 0x57A |

LP_WPA_CNTL1 White peak accumulation control register 1

|  | NTSC-M | NTSC-J | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WPA_TRIGGER_LO | 0x0B3 | 0x0B3 | 0x0B3 | 0x0B3 | 0x096 | 0x096 | 0x096 | 0x0B3 | 0x096 |
| WPA_TRIGGER_HI | 0x21B | 0x21B | 0x21B | 0x21B | 0x1C2 | 0x1C2 | 0x1C2 | 0x21B | 0x1C2 |

LP_VERT_LOCKOUT AGC and Clamp loop lockout control register.

|  | NTSC-M | NTSC-J | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LP_LOCKOUT_START | 0x206 | 0x206 | 0x206 | 0x206 | 0x263 | 0x263 | 0x263 | 0x206 | 0x263 |
| LP_LOCKOUT_END | 0x021 | 0x021 | 0x021 | 0x021 | 0x02C | 0x02C | 0x02C | 0x021 | 0x02C |

## 5.3 Chroma Demodulator

CP_PLL_CNTL0 Chroma PLL Control Register 0

|  | NTSC-M | NTSC-J | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CH_DTO_INC | 0x400000 | 0x400000 | 0x400000 | 0x400000 | 0x400000 | 0x400000 | 0x400000 | 0x400000 | 0x3E7A28 |
| CH_PLL_SGAIN | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 5 |
| CH_PLL_FGAIN | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 6 |

CP_AGC_CNTL

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CH_HEIGHT | 0xCD | 0xCD | 0xCD | 0x91 | 0x91 | 0x9C | 0x9C | 0x9C | 0x66 |
| CH_KILL_LEVEL | 0x0C0 | 0xC0 | 0xC0 | 0x8C | 0x8C | 0x90 | 0x90 | 0x90 | 0x60 |
| CH_AGC_ERROR_LIM | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 |
| CH_AGC_FILTER_EN | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CH_AGC_LOOP_SPEED | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

CP_BURST_GAIN Chroma Loop gain control register.

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BDGHNI | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR_BURST_GAIN | 0x7A | 0x71 | 0x7A | 0x7A | 0x7A | 0x7A | 0x7A | 0x7A | 0x1FF |
| CB_BURST_GAIN | 0xAC | 0x9F | 0xAC | 0xAC | 0xAC | 0xAB | 0xAB | 0xAB | 0x1FF |

CP_ACTIVE_GAIN Chroma Loop gain control register.

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRDR_ACTIVE_GAIN | 0x7A | 0x71 | 0x7A | 0x7A | 0x7A | 0x7A | 0x7A | 0x7A | 0x11C |
| CBDB_ ACTIVE _GAIN | 0xAC | 0x9F | 0xAC | 0xAC | 0xAC | 0xAB | 0xAB | 0xAB | 0x15A |

CP_VERT_LOCKOUT Chroma Loop Lockout control register.

|  | NTSC-M | NTSC- Japan | NTSC- 443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP_VERT_LOCKOUT_START | 0x207 | 0x207 | 0x207 | 0x207 | 0x269 | 0x269 | 0x269 | 0x207 | 0x269 |
| CP_VERT_LOCKOUT_END | 0x00E | 0x00E | 0x00E | 0x00E | 0x00E | 0x012 | 0x012 | 0x00E | 0x012 |

## 5.4 VSYNC PLL

VS_DETECTOR_CNTL	 Vertical sync detector control register

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VSYNC_INT_TRIGGER | 0x2AA | 0x2AA | 0x353 | 0x2AA | 0x353 | 0x2B0 | 0x353 | 0x34D | 0x353 |
| VSYNC_INT_HOLD | 0x017 | 0x017 | 0x01C | 0x017 | 0x01C | 0x017 | 0x01C | 0x01C | 0x01C |

VS_BLANKING_CNTL	Vertical blanking control register

|  | NTSC-M | NTSC- Japan | NTSC- 443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VS_FIELD_BLANK_START | 0x206 | 0x206 | 0x206 | 0x206 | 0x26B | 0x26B | 0x26B | 0x206 | 0x26B |
| VS_FIELD_BLANK_END | 0x00a | 0x00a | 0x00a | 0x00a | 0x02a | 0x02a | 0x02a | 0x00a | 0x02a |

VS_FIELD_ID_CNTL Field ID location control register

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VS_FIELD_IDLOCATION | 0x1 | 0x1 | 0x1 | 0x1 | 0x1 | 0x1 | 0x1 | 0x1 | 0x1 |

VS_FRAME_TOTAL

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VS_FRAME_TOTAL | 0x217 | 0x217 | 0x217 | 0x217 | 0x27B | 0x27B | 0x27B | 0x217 | 0x27B |

## 5.5 Clipping Engine

H_ACTIVE_WINDOW

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N- combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H_ACTIVE_START | 0x06B | 0x06B | 0x07E | 0x067 | 0x09A | 0x07D | 0x09A | 0x084 | 0x095 |
| H_ACTIVE_END | 0x363 | 0x363 | 0x42A | 0x363 | 0x439 | 0x439 | 0x439 | 0x363 | 0x439 |

V_ACTIVE_WINDOW

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N Combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V_ACTIVE_START | 0x025 | 0x025 | 0x025 | 0x025 | 0x02E | 0x02E | 0x02E | 0x025 | 0x02E |
| V_ACTIVE_END | 0x204 | 0x204 | 0x204 | 0x204 | 0x269 | 0x269 | 0x269 | 0x204 | 0x269 |

H_VBI_WINDOW

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N Combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H_VBI_WIND_START | 0x064 | 0x064 | 0x064 | 0x064 | 0x084 | 0x084 | 0x084 | 0x064 | 0x084 |
| H_VBI_WIND_END | 0x366 | 0x366 | 0x366 | 0x366 | 0x41F | 0x41F | 0x41F | 0x366 | 0x41F |

V_VBI_WINDOW

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V_VBI_WIND_START | 0x00b | 0x00b | 0x00b | 0x00b | 0x008 | 0x008 | 0x008 | 0x00b | 0x008 |
| V_VBI_WIND_END | 0x024 | 0x024 | 0x024 | 0x024 | 0x02d | 0x02d | 0x02d | 0x024 | 0x02d |

## 5.6 HSYNC PLL

HS_PLINE Hsync PLL Line total register

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HS_LINE_TOTAL | 0x38E | 0x38E | 0x46F | 0x38D | 0x46F | 0x395 | 0x46F | 0x467 | 0x46F |

HS_DTOINC

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HS_DTO_INC | 0x40000 | 0x40000 | 0x40000 | 0x40000 | 0x40000 | 0x40000 | 0x40000 | 0x40000 | 0x3E7A2 |

HS_PLLGAIN

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HS_PLL_SGAIN | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| HS_PLL_FGAIN | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |

HS_GENLOCKDELAY

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEN_LOCK_DELAY | 0x10 | 0x10 | 0x10 | 0x10 | 0x10 | 0x10 | 0x10 | 0x10 | 0x10 |

HS_MINMAXWIDTH

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MIN_PULSE_WIDTH | 0x21 | 0x21 | 0x29 | 0x21 | 0x29 | 0x21 | 0x29 | 0x29 | 0x29 |
| MAX_PULSE_WIDTH | 0x64 | 0x64 | 0x7D | 0x64 | 0x7D | 0x65 | 0x7D | 0x7D | 0x7D |

HS_WINDOW_LIMIT

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WIN_CLOSE_LIMIT | 0x0A0 | 0x0A0 | 0x0C7 | 0x0A0 | 0x0C7 | 0x0A0 | 0x0C7 | 0x0C7 | 0x0C7 |
| WIN_OPEN_LIMIT | 0x1B7 | 0x1B7 | 0x228 | 0x1B7 | 0x228 | 0x1BB | 0x228 | 0x224 | 0x228 |

## 5.7 Timing Generator

SG_BLACK_GATE

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLANK_INT_START | 0x031 | 0x031 | 0x046 | 0x031 | 0x046 | 0x046 | 0x046 | 0x031 | 0x046 |
| BLANK_INT_LENGTH | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F |

SG_SYNCTIP_GATE

|  | NTSC-M | NTSC- Japan | NTSC-443 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SYNC_TIP_START | 0x0372 | 0x0372 | 0x0453 | 0x0371 | 0x0453 | 0x0379 | 0x0453 | 0x044B | 0x0453 |
| SYNC_TIP_LENGTH | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F |

SG_UVGATE_GATE

|  | NTSC-M | NTSC- Japan | NTSC-433 | PAL-M | PAL-N | PAL-N combination | PAL- BGHIDKK1L | PAL-60 | SECAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UV_INT_START | 0x03B | 0x03B | 0x052 | 0x03B | 0x052 | 0x03B | 0x052 | 0x03C | 0x068 |
| U_INT_LENGTH | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F |
| V_INT_LENGTH | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F | 0x0F |